Shingleback Flight State Machine

UTS Rocketry — Odin Flight Controller

Version 1.0  |  AURC 2026

# Overview

The Shingleback flight computer uses an 8-state finite state machine to manage the full flight sequence from power-on through landing. All pyrotechnic events are gated behind multiple safety interlocks that check altitude, velocity, and acceleration before any charge can fire.

# States

## STATE_IDLE

The default power-on state. The system initialises all peripherals, runs sensor calibration, and waits for an arm command from the ground station over LoRa. Safe for bench testing — no pyro channels are active.

## STATE_PAD

The rocket is armed and sitting on the launch rail. The system continuously monitors the H3LIS331DL high-g accelerometer to detect launches. Pyro channels are armed in software but remain locked until the altitude interlock clears.

## STATE_BOOST

Motor is burning. The system logs all sensor data at the maximum rate. Pyro channels are hard-locked during this state — no ejection charges can fire, regardless of other conditions, while boost is active.

## STATE_COAST

The motor has burned out. The rocket is still climbing under momentum. The Kalman filter tracks vertical velocity — transition to apogee is triggered when the estimated vertical velocity crosses zero.

## STATE_APOGEE

Peak altitude reached. Drogue ejection charge fires. The system verifies the event via IMU settling before transitioning to the drogue state.

## STATE_DROGUE

Rocket is descending under drogue. The system monitors altitude using a Kalman filter and waits for the parafoil deployment altitude threshold to be reached.

## STATE_PARAFOIL

Parafoil is deployed. The CAS board takes control of the steering servos for guided descent. The system monitors altitude and accelerometer data to detect landing.

## STATE_LAND

Rocket has landed. The system transmits GPS position over LoRa at a reduced rate, activates the buzzer for recovery location, and logs final flight data to the SD card.

# Transition Conditions

| From | To | Condition |
| --- | --- | --- |
| STATE_IDLE | STATE_PAD | LoRa arm command received from the ground station |
| STATE_PAD | STATE_BOOST | H3LIS accel > LAUNCH_ACCEL_THRESHOLD (~3g) |
| STATE_BOOST | STATE_COAST | Accel drops back to ~1g (motor burnout) |
| STATE_COAST | STATE_APOGEE | Kalman estimated vertical velocity <= 0 |
| STATE_APOGEE | STATE_DROGUE | Drogue charge fired + descent confirmed |
| STATE_DROGUE | STATE_PARAFOIL | Altitude < PARAFOIL_DEPLOY_ALT (300m AGL) |
| STATE_PARAFOIL | STATE_LAND | Altitude < LAND_ALT_THRESHOLD AND accel settled |

# Pyro Safety Interlocks

Pyrotechnic channels are controlled by Pyro_SafeToFire(), which must return true before any charge can be triggered. ALL of the following conditions must be met simultaneously:

- Current state is NOT STATE_BOOST

- Altitude is above PYRO_ARM_ALT_THRESHOLD (100m AGL minimum)

- Vertical velocity is below MAX_SAFE_VEL

- Acceleration magnitude is below MAX_SAFE_ACCEL

- pyro_armed flag is set (only set in STATE_PAD and above)

The altitude interlock is the primary ground safety mechanism. Even if the software enters an unexpected state on the pad, no pyro channel can fire below 100m AGL. This protects the ground crew during preparation and on the launch rail.

bool Pyro_SafeToFire(void) {

if (current_state == STATE_BOOST)          return false;

if (altitude < PYRO_ARM_ALT_THRESHOLD)     return false;  // 100m AGL

if (vertical_velocity > MAX_SAFE_VEL)      return false;

if (accel_magnitude > MAX_SAFE_ACCEL)      return false;

if (!pyro_armed)                           return false;

return true;

}

| Parameter | Value | Notes |
| --- | --- | --- |
| PYRO_ARM_ALT_THRESHOLD | 100.0 m AGL | Minimum altitude for any pyro event |
| MAX_SAFE_VEL | TBD m/s | Set after motor burn characterisation |
| MAX_SAFE_ACCEL | TBD g | Prevents firing under boost loads |
| pyro_armed | Set in PAD | Cleared on landing |

# Configuration Constants

Defined in flight_state.h:

#define LAUNCH_ACCEL_THRESHOLD    3.0f    // g — triggers PAD to BOOST

#define BURNOUT_ACCEL_THRESHOLD   1.5f    // g — triggers BOOST to COAST

#define PYRO_ARM_ALT_THRESHOLD    100.0f  // m AGL — minimum pyro arm altitude

#define PARAFOIL_DEPLOY_ALT       300.0f  // m AGL — parafoil deployment

#define LAND_ALT_THRESHOLD        10.0f   // m AGL — landing detection

#define LAND_ACCEL_SETTLE         0.1f    // g variance — accel settle threshold

# Implementation Notes

- The state machine runs in the 10ms IMU task loop

- State is stored as a global FlightState_t current_state

- All state transitions are logged to the SD card with a timestamp

- The ground station receives the current state via LoRa telemetry every 200ms

- STATE_IDLE to STATE_PAD is the only transition triggered by LoRa — all others are sensor-driven

- The Kalman filter altitude and velocity estimates are used for all flight phase decisions

# Files

| File | Purpose |
| --- | --- |
| App/ApplicationLayer/FlightStateMachine/flight_state.h | State enum, constants, declarations |
| App/ApplicationLayer/FlightStateMachine/flight_state.c | State machine logic, transitions |
| App/ApplicationLayer/ControlLoop/kalman.h/.c | Kalman filter for altitude/velocity |
| App/AppDrivers/LoRa/LoRa.h/.c | LoRa arm command parsing |